⚡

LIGHTNING PROTECTION

SYSTEM CALCULATOR

Product Requirements Document

Version 1.0  ·  June 2026

Status: Product Requirements Document

| Field | Details |  |
| --- | --- | --- |
| Document Owner | Owuama Chinemere Eberechukwu |  |
| Audience | Developers, QA, Stakeholders, Field Engineers |  |
| Standard | IEC 62305 / BS EN 62305 (Rolling Sphere & Mesh Methods) |  |
| Platform | Web Application (browser-based, responsive) |  |

# 1. Executive Summary

The Lightning Protection System Calculator is a professional web application that enables electrical engineers, building surveyors, and protection system designers to accurately design and specify IEC 62305-compliant lightning protection systems for buildings of any size.

The tool accepts building geometry and system parameters as inputs and produces a complete design output: the total number of lightning arrestors required, optimal placement on the roof, separation distances, protection radii, and full design recommendations — all conforming to international standards.

A built-in map integration (OpenStreetMap / GPS) allows users to pin the exact building location, enabling future extensions such as keraunic level lookup (ground flash density) and site-specific risk assessment.

## 1.1  Problem Statement

Lightning protection design is currently performed manually by engineers using spreadsheets or hand calculations. This process is error-prone, time-consuming, and inconsistent across practitioners. There is no freely available, standards-compliant digital tool that combines geometry input, system selection, automatic calculation, and visual placement output in a single workflow.

## 1.2  Proposed Solution

A browser-based calculator that:

- Accepts building dimensions (L × B × H) and LPS parameters as structured inputs

- Applies the IEC 62305 rolling sphere method and mesh method algorithmically

- Outputs total arrestor count, separation distance, protection radius, and coverage area

- Renders a scaled roof-plan diagram showing each arrestor's optimal position

- Integrates location mapping via address search or GPS for site identification

- Generates actionable engineering recommendations per the selected LPL

# 2. Goals & Success Metrics

## 2.1  Business Goals

- Reduce lightning protection design time from hours to under 5 minutes

- Provide a free, authoritative reference tool for engineers and contractors

- Establish a foundation for a premium SaaS offering with report export and project management

- Support adoption of IEC 62305 standards in markets where hand-calculation is still prevalent

## 2.2  User Goals

- Quickly validate or generate a lightning protection layout for a given building

- Obtain a standards-compliant output suitable for inclusion in design documentation

- Understand exactly how many arrestors are needed and where to place them

- Identify the building on a map for site records and coordination

## 2.3  Non-Goals (v1.0)

- This version does not produce a stamped engineering certificate

- No user accounts, project saving, or cloud storage in v1.0

- No automatic keraunic level / ground flash density (Ng) lookup (planned for v1.1)

- No PDF/Word report export in v1.0 (roadmap item)

- No mobile-native app (web responsive only)

## 2.4  Success Metrics

| Metric | Target (3 months) | Target (12 months) |
| --- | --- | --- |
| Monthly active users | 500+ | 5,000+ |
| Calculation-to-result completion rate | > 80% | > 88% |
| Average session time | 3–7 min | 4–8 min |
| Error / invalid input rate | < 15% | < 10% |
| CSAT / user rating | 4.0 / 5.0 | 4.4 / 5.0 |

# 3. User Personas

## 3.1  Primary Persona — Field Electrical Engineer

Name: Chukwuemeka O. | Role: Electrical Engineer, Lagos | Experience: 8 years

- Designs LPS for commercial buildings, hospitals, and telecoms towers

- Currently performs IEC 62305 calculations manually using Excel

- Needs a fast, trustworthy tool to verify calculations on-site or in the office

- Pain point: re-doing calculations when building dimensions change late in design

## 3.2  Secondary Persona — Building Surveyor

Name: Sarah M. | Role: Quantity Surveyor / Project Manager | Experience: 5 years

- Coordinates LPS procurement; does not perform calculations herself

- Needs to quickly check that a contractor's proposal is plausible

- Values clear visual output (plan view diagram) over technical numbers

## 3.3  Tertiary Persona — LPS Contractor / Installer

Name: Bello A. | Role: LPS Installation Contractor | Experience: 12 years

- Uses the tool to prepare preliminary material lists and quotations

- Needs total arrestor count and placement to estimate cable runs

- Less familiar with IEC standards; values auto-applied defaults

# 4. Functional Requirements

NOTE: Requirements are tagged P0 (must-have v1.0), P1 (should-have v1.0), P2 (nice-to-have / v1.1).

## 4.1  Location Module

| ID | Requirement | Acceptance Criterion | Priority |
| --- | --- | --- | --- |
| L-01 | Address / place-name search field accepting free text input | Returns map pin within 5 s on broadband | P0 |
| L-02 | GPS / geolocation button that reads device coordinates | Pin placed within 50 m of true location | P0 |
| L-03 | Embedded map renders at the resolved location with a visible marker | OpenStreetMap tile loads; marker visible | P0 |
| L-04 | Resolved address / coordinates displayed as human-readable text below the search bar | Text updates on each new search | P0 |
| L-05 | Graceful error message when location not found or GPS denied | Error message shown; app remains usable | P0 |
| L-06 | Latitude / longitude values stored in form state for future report export | Values accessible in output state | P1 |

## 4.2  Building Dimensions Input

| ID | Requirement | Acceptance Criterion | Priority |
| --- | --- | --- | --- |
| D-01 | Numeric input field for building Length (L) in metres | Accepts positive decimals; rejects letters | P0 |
| D-02 | Numeric input field for building Breadth / Width (B) in metres | Accepts positive decimals; rejects letters | P0 |
| D-03 | Numeric input field for building Height (H) in metres | Accepts positive decimals; rejects letters | P0 |
| D-04 | All dimension fields display unit label 'metres' adjacent to input | Label visible on all viewport widths | P0 |
| D-05 | Validation prevents calculation if any dimension is zero or empty | Alert displayed; calculation blocked | P0 |
| D-06 | Height > 20 m triggers a side-flash warning in recommendations | Warning appears in rec. list when H > 20 | P0 |

## 4.3  Protection System Parameters

| ID | Requirement | Acceptance Criterion | Priority |
| --- | --- | --- | --- |
| P-01 | Dropdown for Lightning Protection Level: LPL I, LPL II, LPL III | Default = LPL II | P0 |
| P-02 | Dropdown for system type: Franklin Rods, ESE, Mesh / Faraday Cage | All three options selectable | P0 |
| P-03 | Numeric input for arrestor height above roof (m); default 0.5 m | Pre-populated with 0.5 m on load | P0 |
| P-04 | Contextual secondary input that adapts label/unit based on system type: ESE advance time (µs) ¦ rod tip height (m) ¦ mesh spacing (m) | Label and unit update on system-type change | P0 |
| P-05 | Info bar below parameters showing rolling sphere radius and max separation for selected LPL | Updates on LPL change without recalculation | P0 |

## 4.4  Calculation Engine

The calculation engine must implement the following methods from IEC 62305-3:

### 4.4.1  Rolling Sphere Method (Franklin Rods & ESE)

Protection radius formula:

r_p = √(R² − (R − h)²)

Where R = rolling sphere radius (LPL-dependent) and h = arrestor height above roof.

For ESE systems, the effective protection radius is extended by:

r_esc = r_p + 2·Δt·v_leader   (Δt capped at 120 µs)

| LPL | Rolling Sphere R (m) | Max Separation (m) | Mesh Size (m) |
| --- | --- | --- | --- |
| I | 20 | 20 | 5 |
| II | 30 | 30 | 10 |
| III | 45 | 45 | 15 |

### 4.4.2  Mesh Method (Faraday Cage)

For mesh systems, the mesh size (conductor spacing) must not exceed the IEC 62305-3 limit for the selected LPL. The number of nodes is:

N = (⌈L/s⌉ + 1) × (⌈B/s⌉ + 1)

Where s = mesh size (m). Each node represents a down-conductor connection point.

### 4.4.3  Output Calculations

| Output | Formula / Source | Unit |
| --- | --- | --- |
| Total arrestors (N) | Grid count: (⌈L/sep⌉+1)×(⌈B/sep⌉+1) | integer |
| Protection radius (r_p) | Rolling sphere formula (see 4.4.1) | metres |
| Separation distance (d) | d = 2 × r_p | metres |
| Coverage area per unit | A = π × r_p² | m² |
| Roof area | A_roof = L × B | m² |
| Arrestor height | Direct input — h above roof | metres |

## 4.5  Visual Output — Roof Plan Diagram

| ID | Requirement | Acceptance Criterion | Priority |
| --- | --- | --- | --- |
| V-01 | Scaled top-down (plan view) canvas rendered after calculation | Building rectangle correctly proportioned | P0 |
| V-02 | Each arrestor plotted as a red dot with label (A1, A2, …) | All N arrestors appear on canvas | P0 |
| V-03 | Coverage circles drawn for rod/ESE systems; mesh grid lines drawn for mesh systems | Circles/lines reflect calculated r_p and spacing | P0 |
| V-04 | Dimension labels (L × B in metres) visible on canvas edges | Labels present and readable | P0 |
| V-05 | Legend identifying arrestor dots and coverage zones | Legend below diagram on all viewports | P0 |
| V-06 | Canvas is responsive and does not overflow its container on mobile | Max-width: 100%; height: auto | P1 |
| V-07 | Canvas downloadable as PNG image | Download button produces valid PNG | P2 |

## 4.6  Recommendations Engine

| ID | Requirement | Acceptance Criterion | Priority |
| --- | --- | --- | --- |
| R-01 | System generates ≥ 6 actionable recommendations per calculation | Minimum 6 items in list | P0 |
| R-02 | Recommendations include: arrestor count & type, separation, down-conductors, earthing, conductor specification, SPD | All 6 topics covered | P0 |
| R-03 | Recommendation text adapts to selected LPL and system type (no hard-coded text) | Changing LPL alters at least 3 rec. items | P0 |
| R-04 | Height > 20 m inserts additional side-flash warning recommendation | Extra item appears only when H > 20 | P0 |
| R-05 | All IEC/BS EN 62305 references cited by clause number in recommendations | At least one clause number cited | P1 |

# 5. Non-Functional Requirements

## 5.1  Performance

- Calculation must complete and render in under 500 ms on a standard laptop (Chrome / Firefox)

- Map tile load time is network-dependent; map area must not block form interaction

- Canvas diagram must render within 200 ms of calculation completion

## 5.2  Accuracy & Standards Compliance

- All calculations must conform to IEC 62305-3:2010 and BS EN 62305-3:2011

- Rolling sphere radii, mesh sizes, and separation distances must match Table 2 of IEC 62305-3

- Numeric outputs displayed to 1 decimal place (metres); integer counts as whole numbers

- Floating-point operations must be rounded to prevent display artefacts (e.g., 7.700000001 m)

## 5.3  Usability

- First-time user should be able to complete a calculation without documentation (< 5 min)

- All input fields must have visible labels and unit annotations

- Error states (empty fields, invalid values) must be communicated with clear inline messages

- The tool must be usable on desktop (1280px+) and tablet (768px+) without horizontal scroll

## 5.4  Accessibility

- All interactive elements must be keyboard-navigable (Tab, Enter, Arrow keys)

- Colour contrast ratio must meet WCAG 2.1 AA (minimum 4.5:1 for body text)

- Canvas diagram must have an accessible text summary for screen-reader users

- Map iframe must have a descriptive title attribute

## 5.5  Browser & Device Support

| Browser | Min Version | Support Level |
| --- | --- | --- |
| Google Chrome | 100+ | Primary |
| Mozilla Firefox | 100+ | Primary |
| Microsoft Edge | 100+ | Primary |
| Safari | 15+ | Secondary |
| Mobile Chrome (Android) | 100+ | Secondary |

## 5.6  Security & Privacy

- No user data is transmitted to a backend in v1.0 — all calculation is client-side

- GPS coordinates are used only to centre the map; they are not stored or logged

- Address searches use the Nominatim API (OpenStreetMap); no API key is required

- The application must not request unnecessary device permissions

# 6. Data Dictionary

## 6.1  Inputs

| Field | Label | Unit | Range | Description |
| --- | --- | --- | --- | --- |
| L | Length | m | > 0 | Longest horizontal dimension of building footprint |
| B | Breadth | m | > 0 | Shortest horizontal dimension of building footprint |
| H | Height | m | > 0 | Vertical distance from ground to highest point |
| LPL | Protection Level | – | I, II, III | Lightning Protection Level per IEC 62305-1 |
| SYS | System Type | – | rod / esc / mesh | Type of air termination system |
| h | Arrestor Height | m | > 0.1 | Height of rod tip above roof surface |
| Δt | ESE Advance Time | µs | 1–120 | Time advance of ESE device (ESE systems only) |
| s | Mesh Spacing | m | 1–15 | Conductor spacing for mesh system (≤ LPL limit) |

## 6.2  Outputs

| Output | Unit | Description |
| --- | --- | --- |
| N | integer | Total number of lightning arrestors required |
| r_p | m | Protection radius per arrestor (rolling sphere method) |
| d_sep | m | Separation distance between adjacent arrestors = 2 × r_p |
| A_unit | m² | Coverage area per arrestor = π × r_p² |
| A_roof | m² | Total roof area = L × B |
| Positions | array | Grid coordinates (x,y) for each arrestor on scaled plan |

# 7. User Experience & Flows

## 7.1  Primary User Flow

- User lands on the web app; sees hero header with branding

- User types a building address in the location field or clicks GPS button

- Map renders at the resolved location with a pin

- User enters building Length, Breadth, and Height

- User selects Lightning Protection Level (default: LPL II)

- User selects system type and adjusts arrestor height if required

- User clicks 'Calculate Protection System'

- Results section animates into view showing: metric cards, plan diagram, recommendations

- User reviews placement diagram and recommendations

- (v1.1) User clicks 'Export Report' to download PDF or Word document

## 7.2  Error Flow — Missing Input

- User clicks Calculate without filling all dimension fields

- Alert dialog: 'Please enter valid building dimensions.'

- No results section shown; form remains editable

- User completes missing fields and recalculates

## 7.3  Error Flow — Location Not Found

- User types a non-existent or ambiguous address

- Error message appears below search bar: 'Could not find location. Try a more specific address.'

- Map remains in placeholder state

- User may continue to calculation without a location (location is informational only in v1.0)

## 7.4  Screen Layout

Single-page layout with vertical scroll. Sections in order:

- Header bar — logo, product name, standard reference

- Location Module — search field, GPS button, resolved address, map iframe

- Building Dimensions — 3-column grid: Length / Breadth / Height

- System Parameters — LPL dropdown, system type, arrestor height, contextual input

- Info bar — rolling sphere radius and separation distance for selected LPL

- Calculate button — full-width, high-contrast CTA

- Results section (hidden until calculated) — metric cards, plan canvas, recommendations

# 8. Technical Architecture

## 8.1  Technology Stack

| Layer | Technology | Rationale |
| --- | --- | --- |
| Frontend | HTML5 / CSS3 / Vanilla JavaScript | Zero dependencies; fast load; easy hosting |
| Calculation Engine | Client-side JavaScript functions | Instant results; no server latency or cost |
| Visualisation | HTML5 Canvas API | Scalable, exportable, no library required |
| Map | OpenStreetMap + Nominatim geocoding API | Free, no API key, global coverage |
| Styling | CSS Variables + custom design system | Light/dark mode, theming, maintainability |
| Hosting | Static hosting (Netlify / Vercel / GitHub Pages) | Zero backend; CDN delivery worldwide |

## 8.2  Key Modules

- locationModule.js — Nominatim search, Geolocation API, map iframe injection

- calculationEngine.js — LPL constants, rolling sphere formula, ESE extension, mesh grid count, placement grid generator

- visualRenderer.js — Canvas drawing: building rect, coverage circles/grid, arrestor dots, labels, legend

- recommendationsEngine.js — Template-based recommendations with conditional logic for system type, LPL, and height

- formValidator.js — Input validation, error state management

## 8.3  External Dependencies

- Nominatim API (geocoding): https://nominatim.openstreetmap.org/search — free, no auth, rate limit 1 req/s

- OpenStreetMap tiles (via iframe embed): https://www.openstreetmap.org/export/embed.html — free, attribution required

- No npm packages, no build step required in v1.0

## 8.4  Data Flow

- User fills form fields → JavaScript reads values on button click

- Validator checks all required fields; returns error or proceeds

- Calculation engine computes all output values using IEC 62305 formulae

- Renderer draws Canvas from output values

- Recommendations engine generates text array from output values + system type + LPL

- DOM updated: metric cards, canvas, recommendation list all rendered

# 9. Roadmap

| Phase | Timeline | Features |
| --- | --- | --- |
| v1.0 MVP | Month 1–2 | All P0 requirements; Franklin rods, ESE, mesh; map; plan canvas; recommendations |
| v1.1 | Month 3–4 | Keraunic level (Ng) auto-fetch by GPS; risk assessment (Annex A); PDF report export; canvas PNG download |
| v1.2 | Month 5–6 | User accounts; project save/load; multi-building comparison; bill of materials output |
| v2.0 | Month 7–12 | 3D building model input; full IEC 62305-2 risk assessment; surge study integration; API access for third-party tools |

# 10. Open Questions & Assumptions

## 10.1  Open Questions

- Should the tool support IEC 62305-2 risk assessment (Annex A) in v1.0, or defer to v1.1?

- Is Nominatim geocoding sufficient, or should a Google Maps Platform API key be required for better accuracy in dense urban areas?

- Should the mesh system allow custom conductor spacing input, or enforce LPL-standard limits?

- What level of accuracy is required for the plan diagram — purely schematic, or construction-grade scaled output?

- Will the application require multilingual support (English only for v1.0)?

## 10.2  Assumptions

- All calculations assume a flat, rectangular building footprint. Irregular footprints are out of scope for v1.0.

- The rolling sphere radius values from IEC 62305-3 Table 2 (LPL I=20m, II=30m, III=45m) are authoritative and will not be overridden by user input.

- Location input is informational only in v1.0; it does not influence any calculation outcome.

- The tool will be deployed as a static website with no server-side processing.

- Users are assumed to have basic familiarity with lightning protection concepts; the tool is not intended as a teaching resource.

## 10.3  Dependencies & Risks

| Risk | Likelihood | Mitigation |
| --- | --- | --- |
| Nominatim rate-limiting causes search failures | Medium | Add debounce; allow calculation without location; display fallback message |
| Browser blocks GPS / geolocation | High | Graceful fallback to address search; clear permission prompt |
| Canvas rendering differs across browsers | Low | Test on Chrome, Firefox, Edge, Safari in CI |
| Calculation accuracy challenged by standards body | Low | Document all formulae with IEC clause references; add disclaimer |

# 11. Acceptance Criteria Summary

The following conditions must all be met before v1.0 is considered release-ready:

- All P0 functional requirements (Sections 4.1–4.6) pass QA testing

- Calculation results verified against at least 5 hand-calculated reference cases using IEC 62305-3 examples

- Map search returns correct results for 10 test addresses including Nigerian cities

- GPS location button resolves within 50 m of true position on a physical device

- Plan canvas renders correctly for buildings from 5 m × 5 m to 200 m × 200 m

- No JavaScript errors in browser console during normal usage flows

- Lighthouse performance score ≥ 85 on desktop; ≥ 70 on mobile

- WCAG 2.1 AA colour contrast verified for all text elements

- Cross-browser testing passed on Chrome, Firefox, Edge, and Safari

- Stakeholder sign-off on calculation methodology and recommendation text

BENCHMARKED WITH OTHER LIGHTNING CALCULATOR

https://www.calculatorsconversion.com/en/lightning-protection-radius-rolling-sphere-method-calculator-iec-62305/

https://ecalpro.com/register?callbackUrl=%2Fcalc%2Flightning

https://www.takolightningsystem.com/lightning-protection-calculator/

IEC 63205 Standard

End of Document — Lightning Protection System Calculator PRD v1.0

Prepared June 2026  ·  IEC 62305 / BS EN 62305 Compliant